## Using the self-timer

Use the self-timer when you want to be included in a picture or to ensure a steady Shutter button press. Place the camera on a steady surface.

- Tap the Self-timer/Burst icon, then tap a self-timer setting:

10 seconds - a picture is taken after a 10-second delay (so you have time to get into the scene).

2 seconds - a picture is taken after a 2-second delay (for a steady, shutter release).

2 shot - the first picture is taken after a 10-second delay. A second picture is taken 8 seconds later.

- Compose the scene. Press the Shutter button halfway, then completely down.
The camera takes the picture/pictures after the delay.

Use the same procedure to take a 2- or 10-second self-timer video, but press the Shutter button completely down.

## Using burst

In Burst mode, the camera takes up to 3 pictures while the Shutter button is pressed. Use the same procedure as above to use the burst feature.

- Tap the Self-timer/Burst icon, then tap the Burst icon.

- Compose the scene. Press the Shutter button halfway, then completely down.
The camera stops taking pictures when you release the Shutter button, when the pictures are taken, or when there is no more storage space. Burst is unavailable in some modes.

## Taking panoramic pictures

Use panorama to “stitch” up to 3 pictures into a panoramic scene.

NOTE: Individual pictures are not saved, even if panorama capture is cancelled before stitching occurs. (Exposure Compensation, White Balance, Focus/Exposure Zone, AF Control, Color Mode, and Sharpness are set with the first picture. The flash is set to Off.) You can use the self-timer (2- and 10-second delays) while taking panoramic pictures.

- Tap the Mode icon, then tap SCN.

- Tap to highlight Panorama Left-Right or Panorama Right-Left (depending on the direction you want to pan), then tap X.

- Take the first picture.
After a quickview, the LCD displays the liveview and an “overlay” from the first picture.

- Compose the second picture. Make the overlay from the first picture cover the same portion in the new picture.

- If you are taking/stitching only 2 pictures, tap Stitch to stitch them together.
After you take 3 pictures, the pictures are automatically stitched together.